C     THE CORPORATE

NET-ZERO 2045

Targets, Milestones & Abatement Strategy

Revised — Workbook-Authoritative Numbers

Scope 1+2 Baseline (Location-Based): 193,663 tCO2e

Scope 1+2 Baseline (Market-Based): 166,874 tCO2e

Scope 3 Baseline: 1,401,560 tCO2e

Total Footprint (Market-Based): 1,568,434 tCO2e

Shadow Carbon Price: $100 per tCO2e

Framework: SBTi 1.5 Degrees Celsius Aligned

Data Source: TheCorporate_Emissions_Energy_2023.xlsx

Issued: 16 May 2026

CONFIDENTIAL

1. Executive Summary

This document has been revised to reflect the authoritative numbers from TheCorporate_Emissions_Energy_2023.xlsx. All figures from Section 2 onwards supersede the prior strategy document issued 7 May 2026. The baseline has increased materially: Scope 3 rises from 485,000 to 1,401,560 tCO2e, reflecting the full 15-category GHG Protocol breakdown applied at facility level. Scope 1+2 is broadly consistent at 193,663 tCO2e location-based and 166,874 tCO2e market-based.

The Net-Zero 2045 commitment, SBTi 1.5 degrees Celsius alignment, $100/t shadow carbon price, and 4-wave abatement structure are all unchanged. What changes is the scale: the gross carbon debt to be neutralised by 2045 rises from approximately 1.96M tCO2e to 4.59M tCO2e at 5% annual growth. The SBTi 10% removals cap rises to 458,808 tCO2e — a maximum DAC liability of $91.8M at $200/t.

KEY CHANGE: Total footprint = 1,568,434 tCO2e (market-based). The 2030 SBTi target is now 784,217 tCO2e, not 337,750.

The current approved lever portfolio (approximately 635,000 tCO2e abatement potential) covers 14% of the revised gross carbon debt. Portfolio augmentation is required. Cat 1 purchased goods (728,500 tCO2e, 52% of Scope 3) and Cat 11 product use-phase (320,000 tCO2e, 22.8%) are the two categories that must drive the expanded programme.

2. 2023 Baseline Emissions Inventory

2.1  Scope 1 & 2 — Operational Emissions

Source: TheCorporate_Emissions_Energy_2023.xlsx, Scope 1 Detail and Scope 2 Detail sheets. Emission factors from the Emission Factors sheet, referencing IPCC AR6, IEA 2022-2023, and national grid regulators. Scope 2 is presented under both location-based (LB) and market-based (MB) methods. Market-based is the primary baseline for all target calculations.

| Facility | Scope 1 (tCO2e) | S2 Loc-Based | S1+S2 Loc-Based | S2 Mkt-Based | S1+S2 Mkt-Based |
| --- | --- | --- | --- | --- | --- |
| Hanoi Hub (Vietnam) | 6,175 | 28,629 | 34,804 | 24,680 | 30,855 |
| Guadalajara Gigafactory (Mexico) | 3,132 | 32,688 | 35,820 | 22,700 | 25,832 |
| Shenzhen Systems (China) | 7,966 | 54,614 | 62,580 | 51,128 | 59,094 |
| Wroclaw Precision (Poland) | 4,344 | 22,506 | 26,850 | 15,972 | 20,316 |
| Chennai Circuitry (India) | 4,582 | 29,028 | 33,610 | 26,196 | 30,778 |
| TOTAL | 26,198 | 167,465 | 193,663 | 140,676 | 166,874 |

Scope 2 (LB) is 167,465 tCO2e. Scope 2 (MB) is 140,676 tCO2e. The 26,789 tCO2e difference reflects 49,000 MWh of renewable electricity coverage (16.6% of total consumption) zero-rated under the market-based method. Shenzhen Systems dominates at 54,614 tCO2e (LB) due to volume (94,000 MWh) and China's high grid factor (0.581 tCO2e/MWh). Chennai carries 680 tonnes of stationary diesel — 1,822 tCO2e of avoidable Scope 1 emissions — directly caused by grid instability.

2.2  Scope 3 — Value Chain Emissions by Category

Source: Scope 3 Detail sheet. Full 15-category GHG Protocol breakdown. Total: 1,401,560 tCO2e. Two categories account for 74.8% of the total.

| GHG Protocol Category | Total (tCO2e) | % of Scope 3 |
| --- | --- | --- |
| Cat 1 — Purchased Goods & Services | 728,500 | 52.0% |
| Cat 2 — Capital Goods | 51,200 | 3.7% |
| Cat 3 — Fuel & Energy-Related | 27,600 | 2.0% |
| Cat 4 — Upstream Transportation | 132,900 | 9.5% |
| Cat 5 — Waste Generated in Ops | 7,020 | 0.5% |
| Cat 6 — Business Travel | 2,530 | 0.2% |
| Cat 7 — Employee Commuting | 20,200 | 1.4% |
| Cat 8 — Upstream Leased Assets | 2,800 | 0.2% |
| Cat 9 — Downstream Transport | 59,900 | 4.3% |
| Cat 11 — Use of Sold Products | 320,000 | 22.8% |
| Cat 12 — End-of-Life Treatment | 47,800 | 3.4% |
| Cat 13 — Downstream Leased Assets | 1,110 | 0.1% |
| SCOPE 3 TOTAL | 1,401,560 | 100% |

Cat 1 (Purchased Goods, 728,500 tCO2e) reflects the emissions intensity of aluminium, polymers, and processed metal inputs. Cat 11 (Use of Sold Products, 320,000 tCO2e) reflects in-use electricity consumption of chargers and battery packs across their operational lifetime — a downstream category that requires product design intervention, which falls outside the current lever portfolio and requires a dedicated R&D workstream.

Scope 3 by Facility

Disaggregation of the 5 highest-volume categories by facility, with all remaining categories combined.

| Category | Hanoi | Guadalajara | Shenzhen | Wroclaw | Chennai |
| --- | --- | --- | --- | --- | --- |
| Cat 1: Purchased Goods | 128,400 | 89,200 | 312,000 | 42,100 | 156,800 |
| Cat 4: Upstream Transport | 22,100 | 18,600 | 48,200 | 12,400 | 31,600 |
| Cat 11: Use of Products | 42,000 | 68,000 | 110,000 | 12,000 | 88,000 |
| Cat 12: End-of-Life | 6,400 | 8,200 | 18,400 | 2,200 | 12,600 |
| Cat 9: Downst. Transport | 9,800 | 7,200 | 24,600 | 4,100 | 14,200 |
| All other categories | 17,920 | 21,710 | 36,780 | 16,430 | 19,620 |
| TOTAL S3 | 226,620 | 212,910 | 549,980 | 89,230 | 322,820 |

Shenzhen Systems is the dominant Scope 3 source at 549,980 tCO2e (39.2% of total). Chennai Circuitry ranks second at 322,820 tCO2e. These two facilities are the primary targets for supplier engagement and material circularity programmes.

2.3  Combined Baseline Summary

| Total Scope 1 26,198 tCO2e | Total Scope 2 (LB) 167,465 tCO2e | Total Scope 2 (MB) 140,676 tCO2e | Total Scope 3 1,401,560 tCO2e |
| --- | --- | --- | --- |
| All Scopes — Location-Based 1,595,223 tCO2e | All Scopes — Market-Based 1,568,434 tCO2e |

The primary reporting baseline for all target calculations is the market-based total of 1,568,434 tCO2e. This is the figure against which the SBTi 50% reduction by 2030 is measured.

3. Carbon Debt Engine

Net-Zero requires abating the 2023 footprint plus all emissions generated by business growth to 2045. At 5% annual growth over 22 years, the gross carbon debt is 4,588,079 tCO2e — approximately 2.9 times the 2023 baseline.

| 2023 Baseline (Market-Based) 1,568,434 tCO2e | Growth Delta (5% x 22 yr) 3,019,645 tCO2e |
| --- | --- |
| Gross Carbon Debt to 2045 4,588,079 tCO2e | SBTi Removals Cap (10%) 458,808 tCO2e |

Formula: Total Debt = Baseline + (Baseline x ((1 + r) to the power of n - 1)), where r = 0.05 and n = 22. The growth rate is reviewed biennially. Growth above 7% p.a. sustained for more than 3 years triggers a mandatory portfolio expansion review.

The SBTi removals cap is 458,808 tCO2e. At $200/t, the maximum DAC liability is $91.8M — a 5.4x increase versus the prior estimate. Every tonne of operational abatement achieved displaces $200 of future DAC liability. This makes aggressive Wave 1 deployment the highest-return financial decision in the programme.

4. Reduction Targets & Interim Milestones

4.1  Milestone Schedule

All milestones calculated against the market-based baseline of 1,568,434 tCO2e. The 2030 target of 784,217 tCO2e is the non-negotiable SBTi compliance gate.

| Year | Target (tCO2e) | vs 2023 | Status | Strategic objective |
| --- | --- | --- | --- | --- |
| 2023 | 1,568,434 | Baseline | Starting point | 2023 verified inventory. Market-based baseline. |
| 2027 | 1,254,747 | -20% | Wave 1 complete | Wave 1 complete. Efficiency, solar, modal shift deployed. |
| 2030 | 784,217 | -50% | SBTi 1.5°C gate | SBTi 1.5°C mandatory gate. External verification required. |
| 2035 | 392,109 | -75% | Wave 3 complete | Wave 3 complete. Supply chain transformation embedded. |
| 2040 | 156,843 | -90% | Wave 4 begins | Wave 4 begins. DAC contracts executed for residual only. |
| 2045 | - | -100% | Net-Zero achieved | Net-Zero achieved. All residual neutralised via DAC. |

4.2  Year-on-Year Reduction Rates

The reduction profile is front-loaded. Wave 1 deploys all negative-MAC levers immediately, driving a 7.2% annual rate. Wave 2 sustains momentum through capital-intensive supply chain transformation. The rate moderates in Wave 3 as the residual base shrinks.

| Period | YoY Rate | Abatement Volume | Primary Drivers |
| --- | --- | --- | --- |
| 2025-2027 | -7.2% p.a. | 313,687 tCO2e | LED, HVAC, solar PV, EV forklifts, air-to-sea logistics shift |
| 2028-2032 | -9.8% p.a. | 470,530 tCO2e | Virtual PPAs, heat pumps, recycled aluminium, bio-packaging, supplier mandates |
| 2033-2040 | -5.1% p.a. | 235,266 tCO2e | Supplier SBTi mandates, grid decarbonisation, process electrification |
| 2041-2045 | -7.0% p.a. | 156,843 tCO2e | DAC removals. Residual neutralisation only. $200/t liability. |

The compound annual reduction rate required across the full 22-year programme is 5.8% per annum against a growing baseline. The current lever portfolio covers 14% of gross debt. Portfolio augmentation targeting Cat 1 and Cat 11 is required to close the gap without breaching the removals cap.

5. Abatement Lever Portfolio

5.1  Investment Wave Structure

| Wave 1 2025-2027 7 levers. All MAC < $0. Efficiency, solar, logistics. Rapid payback. 230,000 tCO2e | Wave 2 2028-2032 5 levers. MAC $20-$85. Heavy CAPEX, supply chain transformation. 400,000 tCO2e | Wave 3 2033-2040 1 lever. MAC = $100. Supplier mandates. Threshold monitoring required. 55,000 tCO2e | Wave 4 2040-2045 DAC removals. $200/t. SBTi cap: 458,808 tCO2e max. <=458,808 tCO2e |
| --- | --- | --- | --- |

5.2  Full Lever Portfolio

14 levers across 4 waves. DAC ceiling updated to 458,808 tCO2e (10% of revised gross debt). All other lever potentials are unchanged from the validated technical brief.

| Lever | Scope | Max Abatement | CAPEX ($M) | MAC ($/t) | Wave / Status |
| --- | --- | --- | --- | --- | --- |
| Logistics Modal Shift (Air to Sea) | 3 | 90,000 | $0 | -30 | W1 GO |
| LED Retrofit & Lighting Controls | 1&2 | 15,000 | $0.45 | -45 | W1 GO |
| Compressed Air Leak Detection | 1&2 | 12,000 | $0.15 | -65 | W1 GO |
| HVAC Optimization & AI Sensors | 1&2 | 25,000 | $1.2 | -25 | W1 GO |
| Smart Building Management System | 1&2 | 33,000 | $2.1 | -15 | W1 GO |
| On-site Rooftop Solar PV | 2 | 45,000 | $5.4 | $5 | W1 GO |
| Electric Forklift Fleet Transition | 1 | 10,000 | $1.2 | $15 | W1 GO |
| Off-site Virtual PPA (Wind/Solar) | 2 | 70,000 | $0 | $20 | W2 GO |
| Supplier Energy Efficiency Program | 3 | 75,000 | $0.5 | $25 | W2 GO |
| Bio-based Polymers for Packaging | 3 | 80,000 | $0 | $55 | W2 GO |
| 90% Recycled Aluminium Sourcing | 3 | 140,000 | $0 | $80 | W2 GO |
| Industrial Heat Pumps | 1 | 35,000 | $6.5 | $85 | W2 GO |
| Supplier Renewable Mandate (SBTi) | 3 | 55,000 | $0 | $100 | W3 WATCH |
| Permanent Carbon Removals (DAC) | Rem | 458,808 | $0 | $200 | W4 LAST RESORT |

5.3  Portfolio Gap Analysis

Total operational abatement capacity (Waves 1-3): approximately 635,000 tCO2e. Gross carbon debt: 4,588,079 tCO2e. Gap after all levers and full DAC allowance: approximately 3,494,271 tCO2e. This gap requires portfolio expansion. Recommended additional lever categories:

- Product energy efficiency redesign — targeting Cat 11 (320,000 tCO2e). Reduction of charger and battery in-use consumption through design.

- Extended Tier 2 and Tier 3 supplier engagement — targeting Cat 1 emissions not covered by existing recycled aluminium and bio-polymer levers.

- Product take-back and circular economy programme — addressing Cat 12 end-of-life at scale (47,800 tCO2e).

- M&A carbon screening — any acquisition evaluated against footprint and integration into the Net-Zero roadmap.

5.4  Financial Summary

Wave 1 and 2 approved CAPEX: $23.5M. Annual OPEX savings: $4.47M. Aggregate payback: 5.3 years. DAC liability at full cap: $91.8M. Reducing DAC exposure is the primary financial rationale for accelerating Wave 1.

6. Regional Risk & Regulatory Triggers

Regional modifiers are unchanged in structure. Chennai's 680-tonne stationary diesel load is now explicitly visible in the workbook, confirming grid instability as a material Scope 1 driver.

| Facility | Regulation | Financial Impact | Response Required |
| --- | --- | --- | --- |
| Wroclaw Precision | EU CBAM | $80/t on embodied carbon exported to EU | Accelerate aluminium circularity to Wave 1. |
| Chennai Circuitry | India RPO 2030 | 25% renewable electricity by 2030 | Solar PV is non-discretionary. Deploy by 2028. |
| Hanoi Hub | Grid Instability | 680t diesel on record — 1,822 tCO2e Scope 1 | Battery storage before further electrification. |
| Guadalajara | High Solar IRR | 18% irradiance premium above global average | Priority site for Wave 1 rooftop solar. |

7. SBTi Compliance Guardrails

7.1  Revised 10% Removals Cap

The SBTi removals cap is now 458,808 tCO2e, up from 195,700 in the prior document. Financial liability at the cap: $91.8M. This must be reflected in long-range financial planning.

COMPLIANCE RULE: Removals > 458,808 tCO2e triggers Portfolio Review. Board approval required before any DAC contract is executed.

7.2  2030 Interim Target — Revised

The 2030 SBTi mandatory target is now 784,217 tCO2e (50% of 1,568,434 tCO2e market-based baseline). This replaces the prior figure of 337,750 tCO2e. The absolute reduction required by 2030 — 784,217 tCO2e — exceeds the entire current approved lever portfolio. Reaching this target requires full Wave 1 deployment plus material Wave 2 acceleration, alongside estimated grid decarbonisation contribution of approximately 120,000 tCO2e from passive national grid improvement.

7.3  Scope 2 Method Consistency

The workbook confirms both LB and MB Scope 2 calculations. MB is applied as primary for SBTi reporting. The 49,000 MWh of existing renewable coverage (16.6%) must be preserved and expanded. All PPA and REC contracts require active documentation. Wave 1 solar deployment targets Chennai and Hanoi — the two sites with the lowest existing renewable percentage (9.8% and 13.8% respectively).

8. Governance & Accountability

8.1  Target Ownership

- Chief Sustainability Officer: overall target ownership, annual reporting, SBTi compliance certification.

- Chief Financial Officer: CAPEX approval, DAC liability provisioning ($91.8M ceiling), shadow price governance.

- Chief Operating Officer: facility-level deployment, milestone delivery, contractor management.

- VP Procurement: Scope 3 lever execution — Cat 1 and Cat 4 primary focus. Supplier engagement and annual MAC monitoring.

8.2  Review Cadence

- Annual: Full emissions inventory across all 5 facilities and 15 Scope 3 categories. YoY progress vs targets.

- Biennial: Portfolio rebalancing, growth rate review, MAC recalibration, DAC liability update.

- 2030: External SBTi verification. Target: 784,217 tCO2e. Board ratification required.

- 2040: Wave 4 activation review. Board approval required before any DAC contract is executed.

8.3  Escalation Protocol

If any facility misses its annual abatement target by more than 15%, the COO presents a remediation plan to the CSO within 60 days. If cumulative portfolio performance falls more than 8% behind the milestone schedule, an emergency review convenes within 30 days. Growth above 7% p.a. for 3 consecutive years triggers automatic portfolio expansion review within 90 days.

9. Data Reconciliation

This section documents all differences between the prior strategy document (7 May 2026) and this revised version, and the resolution applied in each case. The workbook is the master data source for all future versions of this document.

| Field | Prior Document | Workbook (Master) | Resolution |
| --- | --- | --- | --- |
| Total S1+S2 (LB) | 190,500 tCO2e | 193,663 tCO2e | Workbook adopted. +3,163 difference within audit tolerance; diesel and mobile combustion now itemised per facility. |
| Total S1+S2 (MB) | 190,500 tCO2e | 166,874 tCO2e | Workbook adopted. Prior doc did not apply MB method. MB is now primary for SBTi reporting. |
| Total Scope 3 | 485,000 tCO2e | 1,401,560 tCO2e | Workbook adopted. Prior doc used 6 aggregated categories. Full 15-cat GHG Protocol now applied. |
| Total Baseline | 675,500 tCO2e | 1,568,434 tCO2e | Workbook adopted (MB). 2.32x higher; driven by Scope 3 reclassification. |
| 2030 SBTi Target | 337,750 tCO2e | 784,217 tCO2e | Recalculated at 50% of revised MB baseline. |
| DAC Removals Cap | 195,700 tCO2e | 458,808 tCO2e | Recalculated at 10% of revised gross carbon debt (4,588,079 tCO2e). |
| Gross Carbon Debt | ~1,957,000 tCO2e | 4,588,079 tCO2e | Recalculated from revised MB baseline at 5% growth over 22 years. |

10. Glossary

| DAC | Direct Air Capture. Removes CO2 directly from the atmosphere and stores it permanently. |
| --- | --- |
| Location-Based S2 | Scope 2 calculated using national grid emission factors. The higher, more conservative figure. |
| Market-Based S2 | Scope 2 calculated using actual PPAs and RECs. PPA-covered electricity = 0 tCO2e/MWh. |
| CAPEX | Capital expenditure. Upfront investment cost. |
| MAC | Marginal Abatement Cost. Net cost (or saving) per tonne abated. Negative = saves money. |
| SBTi | Science Based Targets initiative. Certifies corporate targets as aligned with the Paris Agreement. |
| Shadow Carbon Price | Internal financial value ($100/t) applied to emissions for investment decisions. Not a regulatory tax. |
| Scope 1 | Direct emissions from owned sources: combustion, process heat, company vehicles. |
| Scope 2 | Indirect emissions from purchased electricity, heat, or steam. |
| Scope 3 | All other indirect emissions across 15 GHG Protocol categories. |
| tCO2e | Tonnes of CO2 equivalent. Normalises all greenhouse gases to their CO2 warming potential. |
| CBAM | Carbon Border Adjustment Mechanism. EU levy on high-embodied-carbon imports. |
| PPA | Power Purchase Agreement. Long-term contract to buy electricity from a renewable generator. |
| RPO | Renewable Purchase Obligation. India's legal mandate for minimum renewable electricity. |
| LB / MB | Location-Based / Market-Based. Two GHG Protocol Scope 2 accounting methods. |

The Corporate  -  Net-Zero 2045  -  Revised 16 May 2026  -  Source: TheCorporate_Emissions_Energy_2023.xlsx